City of Toronto

Long-Term Care Homes & Services

2024 Capital Maintenance Plan

Cummer Lodge

MLTC Review Submission

April 2024

Moshsha Charles

Manager, Building Services, Cummer Lodge

Seniors Services and Long-Term Care

City of Toronto

205 Cummer Ave,

North York, ON, M2M 2E8

Tel: 416-392-9506

Moshsha.Charles@toronto.ca

Table of Contents

- Project Summary

  - Maintenance Projects Overview

  - Roofing Replacement – Detailed Description

- Operational Plan

  - Overview of Project

  - Administration

  - Communication

  - General Safety Measures

  - Resident Home Areas (RHA)

  - Kitchen/Food Service

  - Laundry Services

  - Waste Disposal

  - Noise Factors

  - After Hours Work

  - Summary

        - Project Summary

- Existing Home

Cummer Lodge, located in North York, is a Long-Term Care home operated by the City of Toronto and home to 391 residents.

As part of an annual capital maintenance plan, the City plans to conduct several repairs and upgrade projects in order to modernize the facility, and keep the building and grounds in a state of general good repair.

The 2024 Capital Maintenance Plan for Cummer Lodge includes:

- Roof and ductwork insulation repair and replacement:

- Roofing replacement on the South West Pod

- Ductwork and ductwork insulation repair and replacement on entire roof

Work is to begin following Ministry approval of the plans and completion of the City of Toronto tendering process. It is anticipated that the project will take approximately 3 months to complete after the contract is awarded.

- Roofing and Ductwork Insulation Repair and Replacement – Detailed Description

Roofing Work

        - Roofing replacement at the South West Pod; demolition of existing roofing assembly to structural concrete (reinstating undamaged roof ballast and insulation), and installation of new fluid applied roofing assembly.

        - Ductwork and ductwork insulation repair and replacement on entire roof; and pipework insulation repair on entire roof.

Roofing Work (mechanical scope)

c) Ductwork insulation to be replaced with new cladded ductwork insulation. All joints positively lapped and sealed. Work to be done on ductwork from RTU #1-8.

d) Damaged ductwork to be replaced and/or repaired. Leakage test to be performed. Repairs to be done using non-odorous repair methods. Final leakage test to be done to ensure no leaks after ductwork repairs are complete.

e) Damaged insulation on pipework to be repaired and/or replaced.

2) Operational Plan

- Overview of Project

Phasing

This project is anticipated to be done as a single phase at the Home. Roofing replacement and roofing rehabilitation will be performed concurrently or sequentially at the discretion of the General Contractor in order to adhere to the construction timelines.

For additional information regarding construction timelines refer to Anticipated Timelines below.

Permits/Approvals

This project will not require a building permit.

Hours of Work

The following work will take place during regular working hours as defined in the Project Specification and as determined by the Manager of Building Services: 8.00 a.m. to 5.00 pm. and effective sequencing of work must be considered for any unanticipated event.

It is anticipated there will be minimal if any afterhours work required.  The timing of any required work will be coordinated to minimize the impact to residents. All afterhours work shall be at the approval of the Manager of Building Services.

Anticipated Timelines

It is anticipated that this project will begin late June 2024, and be completed by late September 2024.

Special Considerations

Contractor will coordinate area of work with Home.

The project will use cold applied roofing products only - to eliminate any risk of fire that exists with hot applied products and to ensure the safety of residents and staff.

To ensure there is no chance for roofing product fumes to enter the mechanical systems, all rooftop fresh air intakes will be shut down during sensitive activities, be blocked during the work with a carbon/charcoal filter, and materials will be stored away from any air intakes; Refer to mechanical documents. The Home’s mechanical systems will continue to operate in recirculation mode to ensure the air quality of the building will not be compromised.

- Administration

The home’s Administrator will be assisted during this project by the home's staff particularly Manager of Building Services. The Architect for the project is Montgomery Sisam Architects Inc. The City of Toronto’s Head Office Project Manager will also manage the Owner’s responsibilities.

In order to facilitate effective and timely communication the following protocols will be in-place:

- Morning meetings with the Administrator or delegate.

- Administrative rounds with the Supervisor Building Services or delegate and union co-chair of the OH&S Committee as required.

- Communication

Before construction begin, a start-up meeting / orientation will be held with the Contractor and home staff, outlining the requirements imposed on Contractors working in a Long-Term Care environment, and protocols that the Contractor will be expected to follow.  All issues with respect to both resident and staff safety will be identified with staff involvement and will be communicated though formal and informal meetings.

- The Residents’ Council as well as residents and families will be informed of the maintenance projects, expected completion dates and any contingency plan if required.

- All staff will be informed and communicated via general staff meetings, departmental meetings and internal memorandums.

- The Administrator or delegate will notify the Ministry of Health and Long-Term Care Regional Office when work is about to begin.

- General Safety Measures

Name of person assigned to monitor safety:

Moshsha Charles, Manager, Building Services

All areas of work will be separated from building users and visitors using barricades and/or hording as required as to the satisfaction of the Building Services Manager and the Architect.

Each area will be cleaned of debris and construction material frequently during each day and thoroughly cleaned at shift end.

- Barriers / Tool and Equipment Storage

The Contractor must provide temporary measures to ensure all work areas, for all projects, are secure from residents between all working areas and resident spaces.  Dust protection must be incorporated to protect residents and resident spaces from dust exposure.  Tools and equipment must not be left in areas accessible to residents and must be locked in a secure location when unattended. All tools and equipment shall be stored in a secured location as coordinated with the Building Services Manager.

- Alarms

All door alarms, fire alarms and nurse call devices will operate normally in all areas of the home during the renovation period. Should it be necessary to temporarily bypass a fire zone, a manual fire watch will be implemented. The Home will work with the alarm monitoring company as necessary in case of a bypass required. All fire zones will be restored to full operation at the end of each construction day.

- Housekeeping / Cleanliness

The Contractor shall keep all construction areas clean of construction debris and waste throughout the day.  Housekeeping routines will be adjusted to maintain an acceptable level of cleanliness.  Housekeeping hours will be adjusted, if required, to keep up with the increased demand.

- Transporting Construction Materials

Construction deliveries will be delivered to specific area as coordinated with the Building Services Manager.  All materials that are to be transported throughout the building shall be transported by routes as informed by the Building Services Manager. If required to transport goods and equipment to floor levels not accessible from grade level, the Contractor shall use a dedicated elevator and must coordinate with the home timelines for elevator use to cause minimal disturbances to the home.

- Water Shutoff

There are no anticipated works requiring water shut offs.

- Emergency Plan / Life Safety Plan

This work does not affect the overall supply of electricity, natural gas or life safety systems within the Home.

Although work on this project should not cause a disruption to services, if such a disruption were to occur the Home follow its Emergency Management Plan. The plan has measures that compensate for the loss of electricity or natural gas supply. In order to ensure the smooth implementation of the Emergency Management Plan, the Administrator, and the Manager Building Services will review the plan and ensure staff are able to meet it, prior to work commencing on the project.

- Resident Home Areas (RHA)

Since the work will take place in areas not occupied by residents, it is anticipated that there will be no impact to resident areas

- Kitchen/Food Service

There will be no impact to food services.

Delivery of food and any other supplies required to provide care to the residents will not be affected due to this project.

- Laundry Service

No impact on laundry service throughout the facility.

- Waste Disposal

Contractor to put a waste bin within the loading area of Long-Term Care Home. Contractor should erect a signage to make sure no mixing on construction and home waste.

- Noise Factors

Construction will take place during the normal business hours of 8:00 a.m. – 5:00 p.m. The roofing work occurring above resident areas is anticipated to be solely rehabilitation, therefore there will be no anticipated noise impact. If drilling is required above resident rooms, Contractor to coordinate timing of work with Home to reduce disruptions.

- After Hours Work

It is anticipated there will be minimal if any afterhours work required.  The timing of any required work will be coordinated to minimize the impact to residents. All afterhours work shall be at the approval of the Manager of Building Services.

- Summary

The work outlined in this operational plan will require co-ordination and communication between the Contractor and the Home to ensure the planned improvements are achieved with minimal impact to residents.

When preparing this Operational Plan an emphasis was placed on the resident and staff's comfort and safety. This plan works to minimize impact on the daily lives of the people living and working at Cummer Lodge by holding Contractors to their schedules and maintaining clear lines of communication between all staff members, residents and families.